# SYSTEM ON CHIP SPECIFICATION

# 16-BIT RISC MCU WITH UART

THOMAS NGUYEN
CECS 460
14 MAY 2019

DEPARTMENT OF COMPUTER ENGINEERING AND COMPUTER SCIENCE

This document contains information proprietary to the CSULB student that created the file – any reuse without adequate approval and documentation is prohibited

In submitting this file for class work at CSULB I am confirming that this is my work and the work of no one else.

In the event, other code sources are used, I will give document and give credit to the portion of code belonging to the author.

In submitting this project, I acknowledge that plagiarism in student project work is subject to dismissal from the class.

Copyright © 2019 Thomas Nguyen. All rights reserved.

INTENTIONALLY BLANK

Table Contents

SECTION 1: INTRODUCTION	10

1.1: PURPOSE	10

SECTION 2: DOCUMENTS	11

2.1: NEXYS 4 BOARD DATASHEET	11

2.1.1: UART CONNECTIONS	11

2.1.2: I/O Connections	12

2.2: ARTIX-7 LIBRARY	13

2.3: PICOBLAZE MICROPROCESSOR	13

2.3.1: ARCHITECTURE	14

2.3.2: INSTRUCTION SET	14

SECTION 3: REQUIREMENTS	17

3.1: INTERFACE REQUIREMENTS	17

3.2: PHYSICAL REQUIREMENTS	18

SECTION 4: TOP LEVEL IMPLEMENTATION	19

4.1: DESCRIPTION	19

4.2: BLOCK DIAGRAM	20

4.3: DATA FLOW DESCRIPTION	20

4.4: INPUT/OUTPUT	21

4.4.1: SIGNAL NAMES	21

4.4.2: PIN ASSIGNMENTS	21

4.4.3: ELECTRICAL CHARACTERISTICS	21

4.5: CLOCKS	21

4.6: RESETS	22

4.7: SOFTWARE	22

4.7.1: DESCRIPTION	22

4.7.2: SOFTWARE PLANNING DOCUMENT	22

4.7.3: SOURCE CODE	26

SECTION 5: EXTERNALLY ACQUIRED BLOCKS	27

5.1: TRAMELBLAZE	27

5.1.1: DESCRIPTION	27

5.1.2: FUNCTIONAL REQUIREMENTS	27

5.1.3: BLOCK DIAGRAM	27

5.1.4: INPUT/OUTPUT	27

SECTION 6: INTERNALLY DEVELOPED BLOCKS	28

6.1: TSI	28

6.1.1: DESCRIPTION	28

6.1.2: BLOCK DIAGRAM	28

6.1.3: INPUT/OUTPUT	28

6.1.4: SOURCE CODE	28

6.2: SOPC_CORE	29

6.2.1: DESCRIPTION	29

6.2.2: BLOCK DIAGRAM	31

6.2.3: INPUT/OUTPUT	32

6.2.4: VERIFICATION	32

6.2.5: SOURCE CODE	32

6.3: RECEIVE ENGINE	33

6.3.1: DESCRIPTION	33

6.3.2: BLOCK DIAGRAM	34

6.3.3: INPUT/OUTPUT	35

6.3.4: VERIFICATION	35

6.3.5: SOURCE CODE	35

6.3.6: RECEIVE ENGINE CONTROL UNIT	36

6.3.7: RECEIVE ENGINE DATAPATH	39

6.4: TRANSMIT ENGINE	45

6.4.1: DESCRIPTION	45

6.4.2: BLOCK DIAGRAM	46

6.4.3: INPUT/OUTPUT	47

6.4.4: VERIFICATION	47

6.4.5: SOURCE CODE	47

6.4.6: SHIFT REGISTER	48

6.5: EMBEDDED STATIC RAM	50

6.5.1: DESCRIPTION	50

6.5.2: BLOCK DIAGRAM	50

6.5.3: INPUT/OUTPUT	51

6.5.4: VERIFICATION	51

6.6: ASYNCHRONOUS-IN SYNCHRONOUS-OUT	53

6.6.1: DEFINITION	53

6.6.2: BLOCK DIAGRAM	53

6.6.3: INPUT/OUTPUT	53

6.6.4: VERIFICATION	53

6.6.5: SOURCE CODE	53

6.7: LOAD REGISTER	54

6.7.1: DEFINITION	54

6.7.2: BLOCK DIAGRAM	54

6.7.3: INPUT/OUTPUT	54

6.7.4: VERIFICATION	54

6.7.5: SOURCE CODE	54

6.8: POSEDGE DETECT	55

6.8.1: DEFINITION	55

6.8.2: BLOCK DIAGRAM	55

6.8.3: INPUT/OUTPUT	55

6.8.4: VERIFICATION	55

6.8.5: SOURCE CODE	55

6.9: ADDRESS DECODER	56

6.9.1: DEFINITION	56

6.9.2: BLOCK DIAGRAM	56

6.9.3: INPUT/OUTPUT	56

6.9.4: VERIFICATION	56

6.9.5: SOURCE CODE	56

6.10: BAUD DECODER	57

6.10.1: DEFINITION	57

6.10.2: INPUT/OUTPUT	57

6.10.3: VERIFICATION	57

6.10.4: SOURCE CODE	57

6.9: SR FLOP	58

6.11.1: DEFINITION	58

6.11.2: BLOCK DIAGRAM	58

6.11.3: INPUT/OUTPUT	58

6.11.4: VERIFICATION	58

6.11.5: SOURCE CODE	58

SECTION 7: CHIP VERIFICATION PLAN	59

7.1: SOPC CORE VERIFICATION	59

7.2: RECEIVE ENGINE VERIFICATION	61

7.3: TRANSMIT ENGINE VERIFICATION	62

SECTION 8: CHIP LEVEL TEST	64

APPENDIX:	67

A.1: TOP LEVEL	68

A.2: TECHNOLOGY SPECIFIC INSTATIATIONS	69

A.3: SOPC CORE	70

A.4: ASYNCHRONOUS-IN SYNCHRONOUS-OUT REGISTER	72

A.5: ADDRESS DECODER	73

A.6: SET RESET FLOP	74

A.7: POSITIVE-EDGE DETECT	75

A.8: LOAD REGISTER	76

A.9: BAUD DECODER	77

A.10: TRANSMIT ENGINE	78

A.11: RECEIVE ENGINE	80

A.12: RECEIVE ENGINE CONTROL UNIT	81

A.13: RECEIVE ENGINE DATAPATH	83

A.14: ASSEMBLY PROGRAM	85

Table of Figures

Figure 1: Nexys 4 UART Connection	11

Figure 2: Nexys 4 I/O Connections	12

Figure 3: IBUF/IBUFG	13

Figure 4: OBUF	13

Figure 5: TramelBlaze Architecture	14

Figure 6: Top Level Block Diagram	19

Figure 7: Top Level Detailed Diagram	20

Figure 8: Flowchart for Initialization and Main Loop	22

Figure 9: TramelBlaze Top Level Block Diagram	27

Figure 10: TSI Block Diagram	28

Figure 11: SOPC_CORE Block Design	29

Figure 12: SOPC_CORE Detailed Block Design	31

Figure 13: RX_Engine Block	33

Figure 14: Receive Engine Detailed Block Design	34

Figure 15: RX_control FSM	36

Figure 16: RX_control verification 1	37

Figure 17: RX_control verification 2	38

Figure 18: RX_control verification 3	38

Figure 19: RX_DATAPATH Block Diagram	39

Figure 20: RX_Datapath Detailed Block Design	40

Figure 21: RX_Datapath Verification 1	41

Figure 22: RX_Datapath Verification 2	42

Figure 23: RX_Datapath Verification 3	42

Figure 24: RX Shift Register Detailed Block Diagram	43

Figure 25: RX Shift Register Verification	43

Figure 26: Remap Register Detailed Block Diagram	44

Figure 27: Remap Verification	44

Figure 28: TX_ENGINE Block Diagram	45

Figure 29: TX_ENGINE Detailed Block Diagram	46

Figure 30: Shift Register Block Diagram	48

Figure 31: Shift Register Detailed Block Diagram	48

Figure 32: SRAM Block Diagram	50

Figure 33: SRAM Verification 1	51

Figure 34: SRAM Verification 2	51

Figure 35: SRAM Verification 3	52

Figure 36: SRAM Verification 4	52

Figure 37: SRAM Verification 5	52

Figure 38: AISO Block Detailed Diagram	53

Figure 39: AISO Verification	53

Figure 40: Load Register Block Detailed Diagram	54

Figure 41: Load Register Verification	54

Figure 42: Posedge Detect Detailed Block Diagram	55

Figure 43: Posedge Detect Verification	55

Figure 44: Address Decoder Block Diagram	56

Figure 45: Address Decoder Verification	56

Figure 46: Baud Decoder Block Diagram	57

Figure 47: Baud Decoder Verification	57

Figure 48: SR Flop Block Diagram	58

Figure 49: SR Flop Verification	58

Figure 50: RX_Engine Verification	61

Figure 51: Transmit Engine Verification	63

Figure 52: Program Verification 1	64

Figure 53: Program Verification 2	64

Figure 54: Memory Test 1	65

Figure 55: Memory Test 2	65

Figure 56: Memory Test 3	66

Figure 57: Dump Memory Test	66

Table of Tables

Table 1: TramelBlaze Instruction Set	14

Table 2: Baud Rate	17

Table 3: Parity Control	17

Table 4: Physical Buttons	18

Table 5: Physical Switches	18

Table 6: Physical Light Emitting Diodes	18

Table 7: Top_Level Signal Names	21

Table 8: Top_Level Pin Assignments	21

Table 9: TramelBlaze Signal Names	27

Table 10: TSI Signal Names	28

Table 11: SOPC_CORE Signal Names	32

Table 12: RX Engine Signal Names	35

Table 13: RX_control Signal Names	37

Table 14: RX_Datapath Signal Names	41

Table 15: RX Shift Register Signal Names	43

Table 16: Remap Signal Names	44

Table 17: TX_ENGINE Signal Names	47

Table 18: Shift Register Signal Names	49

Table 19: SRAM Signal Names	51

Table 20: AISO Signal Names	53

Table 21: Load Register Signal Names	54

Table 22: Posedge Detect Signal Names	55

Table 23: Address Decoder Signal Names	56

Table 24: Baud Decoder Signal Names	57

Table 25: SR Flop Signal Names	58

# SECTION 1: INTRODUCTION

The Chip Specification Manual details all the information there is about the Universal Asynchronous Receiver and Transmitter (UART) and the 16-bit embedded system TramelBlaze. The embedded system, TramelBlaze, utilizing the UART is an emulator of the 8-bit PicoBlaze to work on the Nexys 4 Artix-7 FPGA. These two components are abstract layers making the System on a Programmable Chip (SOPC). The SOPC communicates to a specific device utilizing the UART protocol through the Technology Specific Instantiation (TSI) or commonly known as I/O PAD to use the I/O drivers from standard Artix-7 libraries.

## 1.1: PURPOSE

The purpose of this manual is to provide information for every part of a chip in order to recreate, use, and understand the SOPC and TSI. Chip level block diagrams, detailed block diagrams, I/O signal mapping and module verification will be provided to further the understanding of these two components. Utilizing the microprocessor, an assembly program can be used to demonstrate how the SOPC uses the UART protocol to receive or transmit data through the microprocessor and an embedded SRAM to a device terminal.

# SECTION 2: DOCUMENTS

The section entails the documents and hardware used to assist in designing the chip. The Nexys 4, the Artix-7 libraries and the PicoBlaze microprocessor are key interfaces to the SOPC in order to use the assembly program and the UART protocol.

## 2.1: NEXYS 4 BOARD DATASHEET

The datasheet details the specifics to use the Nexys 4 FPGA board for the SOPC. It includes the schematic for the UART connections and the basic I/O connections.

### 2.1.1: UART CONNECTIONS

Figure 1: Nexys 4 UART Connection

Figure 1 shows the necessary connections between the Nexys 4 Board and the Micro-USB to communicate using UART serial communication protocol.

### 2.1.2: I/O Connections

Figure 2: Nexys 4 I/O Connections

Figure 2 shows all the available I/O port connections possible on the Nexys 4 Artix-7 FPGA board. The necessary connections for the SOPC are the buttons, LEDs and slide switches.

## 2.2: ARTIX-7 LIBRARY

The standard library provides the simple essentials to implement the TSI for the SOPC input and output signals.

Figure 3: IBUF/IBUFG

Figure 3 shows an input buffer used for all inputs. The IBUF is the universal input buffer while the IBUFG is for the clock input.

Figure 4: OBUF

Figure 4 shows the universal output buffer for all output. It drives signals from the FPGA to external output pads.

## 2.3: PICOBLAZE MICROPROCESSOR

The PicoBlaze is an 8-bit RISC microprocessor from Xilinx for use in their FPGA and CPLD products. The PicoBlaze is compatible with some of the Xilinx FPGAs. It was made for the Spartan-3, Virtex-II and Virtex-II Pro families. The processor comes with license cores that are free to use for Xilinx products. In order for the PicoBlaze to be compatible with the Artix-7, John Tramel, the professor of CECS 460, developed an emulator to run the PicoBlaze as a 16-bit RISC microprocessor. The PicoBlaze utilizes assembly program instructions inside the instruction ROM to fetch, decode and execute register-based instructions. The PicoBlaze consists of a 1K x 18 PROM, 64 word scratchpad RAM, and 31 word call/return stack. The TramelBlaze ups the memories to 4K x 16 PROM, 512 x 16 scratchpad, 128 x 16 call/return stack. Both have 16 16-bit wide registers.

### 2.3.1: ARCHITECTURE

Figure 5: TramelBlaze Architecture

The PicoBlaze architecture is no different from the TramelBlaze with the exception of the different memory sizes.

### 2.3.2: INSTRUCTION SET

Table 1: TramelBlaze Instruction Set

| Instruction | Description | Function | ZERO | CARRY |
| --- | --- | --- | --- | --- |
| ADD sX, kk | Add register sX with literal kk | sX ← sX + kk | ? | ? |
| ADD sX, sY | Add register sX with register sY | sX ← sX + sY | ? | ? |
| ADDCY sX, kk (ADDC) | Add register sX with literal kk with CARRY bit | sX ← sX + kk + CARRY | ? | ? |
| ADDCY sX, sY (ADDC) | Add register sX with register sY with CARRY bit | sX ← sX + sY + CARRY | ? | ? |
| AND sX, kk | Bitwise AND register sX with literal kk | sX ← sX AND kk | ? | 0 |
| AND sX, sY | Bitwise AND register sX with register sY | sX ← sX AND sY | ? | 0 |
| CALL aaa | Unconditionally call subroutine at aaa | TOS ← PC PC ← aaa | - | - |
| CALL C, aaa | If CARRY flag set, call subroutine at aaa | If CARRY = 1, {TOS ← PC, PC ← aaa} | - | - |
| CALL NC, aaa | If CARRY flag not set, call subroutine at aaa | If CARRY = 0, {TOS ← PC, PC ← aaa} | - | - |
| CALL NZ, aaa | If ZERO flag not set, call subroutine at aaa | If ZERO = 0, {TOS ← PC, PC ← aaa} | - | - |
| CALL Z, aaa | If ZERO flag set, call subroutine at aaa | If ZERO = 1, {TOS ← PC, PC ← aaa} | - | - |
| COMPARE sX, kk (COMP) | Compare register sX with literal kk. Set CARRY and ZERO flags as appropriate. Registers are unaffected. | If sX = kk, ZERO ← 1 If sX < kk, CARRY ← 1 | ? | ? |

Table 1: TramelBlaze Instruction Set (continue)

| Instruction | Description | Function | ZERO | CARRY |
| --- | --- | --- | --- | --- |
| COMPARE sX, sY (COMP) | Compare register sX with register sY. Set CARRY and ZERO flags as appropriate. Registers are unaffected. | If sX = sY, ZERO ← 1 If sX < sY, CARRY ← 1 | ? | ? |
| DISABLE INTERRUPT (DINT) | Disable interrupt input | INTERRUPT_ENABLE ← 0 | - | - |
| Interrupt Event | Asynchronous interrupt input. Preserve flags and PC. Clear INTERRUPT_ENABLE flag. Jump to interrupt vector at address 3FF. | Preserved ZERO ← ZERO Preserved CARRY ← CARRY INTERRUPT_ENABLE ← 0 TOS ← PC PC ← 3FF | - | - |
| FETCH sX, (sY) (FETCH sX, sY) | Read scratchpad RAM location pointed to by register sY into register sX | sX ← RAM[(sY)] | - | - |
| FETCH sX, ss | Read scratchpad RAM location ss into register sX | sX ← RAM[ss] | - | - |
| INPUT sX, pp (IN) | Read value on input port location pp into register sX | PORT_ID ← pp sX ← IN_PORT | - | - |
| JUMP aaa | Unconditionally jump to aaa | PC ← aaa | - | - |
| JUMP C, aaa | If CARRY flag set, jump to aaa | If CARRY = 1, PC ← aaa | - | - |
| JUMP NC, aaa | If CARRY flag not set, jump to aaa | If CARRY = 0, PC ← aaa | - | - |
| JUMP NZ, aaa | If ZERO flag not set, jump to aaa | If ZERO = 0, PC ← aaa | - | - |
| JUMP Z, aaa | If ZERO flag set, jump to aaa | If ZERO = 1, PC ← aaa | - | - |
| LOAD sX, kk | Load register sX with literal kk | sX ← kk | - | - |
| LOAD sX, sY | Load register sX with register sY | sX ← sY | - | - |
| OR sX, kk | Bitwise OR register sX with literal kk | sX ← sX OR kk | ? | 0 |
| OR sX, sY | Bitwise OR register sX with register sY | sX ← sX OR sY | ? | 0 |
| OUTPUT sX, (sY) (OUT sX, sY) | Write register sX to output port location pointed to by register sY | PORT_ID ← sY OUT_PORT ← sX | - | - |
| OUTPUT sX, (pp) (OUT sX, pp) | Write register sX to output port location pp | PORT_ID ← pp OUT_PORT ← pp | - | - |
| RETURN (RET) | Unconditionally return from subroutine | PC ← TOS + 1 | - | - |
| RETURN C (RET C) | If CARRY flag set, return from subroutine | If CARRY = 1, PC ← TOS + 1 | - | - |
| RETURN NC (RET NC) | If CARRY flag not set, return from subroutine | If CARRY = 0, PC ← TOS + 1 | - | - |
| RETURN NZ (RET NZ) | If ZERO flag not set, return from subroutine | If ZERO = 0, PC ← TOS + 1 | - | - |
| RETURN Z (RET Z) | If ZERO flag set, return from subroutine | If ZERO = 1, PC ← TOS + 1 | - | - |
| RETURNI DISABLE (RETI DISABLE) | Return from interrupt service routine. Interrupt remains disabled. | PC ← TOS ZERO ← Preserved ZERO CARRY ← Preserved CARRY INTERRUPT_ENABLE ← 0 | - | - |
| RETURNI ENABLE (RETI ENABLE) | Return from interrupt service routine. Re-enable interrupt. | PC ← TOS ZERO ← Preserved ZERO CARRY ← Preserved CARRY INTERRUPT_ENABLE ← 1 | - | - |
| RL sX | Rotate register sX left | sX ← {sX[6:0], sX[7]} CARRY ← sX[7] | ? | ? |
| RR sX | Rotate register sX right | sX ← {sX[0], sX[7:1]} CARRY ← sX[0] | ? | ? |

Table 1: TramelBlaze Instruction Set (continue)

| Instruction | Description | Function | ZERO | CARRY |
| --- | --- | --- | --- | --- |
| SL0 sX | Shift register sX left, zero fill | sX ← {sX[6:0], 0} CARRY ← sX[7] | ? | ? |
| SL1 sX | Shift register sX left, one fill | sX ← {sX[6:0], 1} CARRY ← sX[7] | 1 | ? |
| SLA sX | Shift register sX left through all bits, including CARRY | sX ← {sX[6:0], CARRY} CARRY ← sX[7] | ? | ? |
| SLX sX | Shift register sX left. Bit sX[0] is unaffected | sX ← {sX[6:0], sX[0]} CARRY ← sX[7] | ? | ? |
| SR0 sX | Shift register sX right, zero fill | sX ← {0, sX[7:1]} CARRY ← sX[0] | ? | ? |
| SR1 sX | Shift register sX right, one fill | sX ← {0, sX[7:1]} CARRY ← sX[0] | 1 | ? |
| SRA sX | Shift register sX right through all bits, including CARRY | sX ← {CARRY, sX[7:1]} CARRY ← sX[0] | ? | ? |
| SRX sX | Shift register sX right. Bit sX[0] is unaffected | sX ← {sX[7], sX[7:1]} CARRY ← sX[0] | ? | ? |
| STORE sX, (sY) (STORE sX, sY) | Write register sX to scratchpad RAM location pointed to by register sY | RAM[(sY)] ← sX | - | - |
| STORE sX, ss | Write register sX to scratchpad RAM location ss | RAM[ss] ← sX | - | - |
| SUB sX, kk | Subtract literal kk from register sX | sX ← sX – kk | - | - |
| SUB sX, sY | Subtract register sY from register sX | sX ← sX – sY | - | - |
| SUBCY sX, kk (SUBC) | Subtract literal kk from register sX with CARRY (borrow) | sX ← sX – kk – CARRY | ? | ? |
| SUBCY sX, sY (SUBC) | Subtract register sY from register sX with CARRY (borrow) | sX ← sX – sY – CARRY | ? | ? |
| TEST sX, kk | Test bits in register sX against literal kk. Update CARRY and ZERO flags. Registers are unaffected | If(sX AND kk) = 0, ZERO ← 1 CARRY ← odd parity of (sX AND kk) | ? | ? |
| TEST sX, sY | Test bits in register sX against register sY. Update CARRY and ZERO flags. Registers are unaffected | If(sX AND sY) = 0, ZERO ← 1 CARRY ← odd parity of (sX AND sY) | ? | ? |
| XOR sX, kk | Bitwise XOR register sX with literal kk | sX ← sX XOR kk | ? | 0 |
| XOR sX, sY | Bitwise XOR register sX with register sY | sX ← sX XOR sY | ? | 0 |

| Operand | Description |
| --- | --- |
| sX | One of 16 possible register locations ranging from s0 through sF or specified as a literal |
| sY | One of 16 possible register locations ranging from s0 through sF or specified as a literal |
| aaa | 12-bit address, specified either as a literal or a four-digit hexadecimal value ranging from 0000 to FFFF or a labeled location. |
| kk | 16-bit immediate constant, specified as a literal or a two-digit hexadecimal value ranging from 00 to FF or specified as a literal |
| pp | 16-bit port address, specified as a literal or a two-digit hexadecimal value ranging from 00 to FF or specified as a literal |
| ss | 9-bit scratchpad RAM address, specified either as a literal or a three-digit hexadecimal value ranging from 000 to 1FF or specified as a literal |
| RAM[n] | Contents of scratchpad RAM at location n |
| TOS | Value stored at top of stack |

# SECTION 3: REQUIREMENTS

## 3.1: INTERFACE REQUIREMENTS

The input and output of the SOPC core comes from and to the Nexys 4 Artix-7 FPGA board through a Technology Specific Instantiation (TSI). To communicate with each other, the design has eleven baud rates to choose from using four onboard switches of the FPGA. Parity is controlled by another three switches. The parity control contains a switch for eight-bit, parity enable and odd/even parity select. These are based on and compatible with industrial standards.

Table 2: Baud Rate

| Switches [7:4] | Bit Time | Baud Rate |
| --- | --- | --- |
| 0000 | 3.3333 ms | 300 |
| 0001 | 833.33 us | 1200 |
| 0010 | 416.66 us | 2400 |
| 0011 | 208.33 us | 4800 |
| 0100 | 104.16 us | 9600 |
| 0101 | 52.083 us | 19200 |
| 0110 | 26.041 us | 38400 |
| 0111 | 17.361 us | 57600 |
| 1000 | 8.6806 us | 115200 |
| 1001 | 4.3403 us | 230400 |
| 1010 | 2.1701 us | 460800 |
| 1011 | 1.0851 us | 921600 |

Table 3: Parity Control

| Switches [3:1] | Bits | Parity |
| --- | --- | --- |
| 000 | 7 | None |
| 001 | 7 | None |
| 010 | 7 | Even |
| 011 | 7 | Odd |
| 100 | 8 | None |
| 101 | 8 | None |
| 110 | 8 | Even |
| 111 | 8 | Odd |

## 3.2: PHYSICAL REQUIREMENTS

The SOPC utilizes seven switches out of sixteen from the FPGA board to control the baud rate and parity and one button out of five for reset. The switches are in a specific order for a register within the SOPC. The switches [7:1] are used. Switches [7:4] is for baud rate control. Switches [3:1] is for parity control. The LED[15:0] are used to show that the board is on and actually running outside of the UART terminal.

Table 4: Physical Buttons

| BTNU | BTND | BTNC | BTNL | BTNR |
| --- | --- | --- | --- | --- |
| SYS_RESET | N/A | N/A | N/A | N/A |

Table 5: Physical Switches

| SW7 | SW6 | SW5 | SW4 | SW3 | SW2 | SW1 | SW0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| i_SW[7] | i_SW[6] | i_SW[5] | i_SW[4] | i_SW[3] | i_SW[2] | i_SW[1] | N/A |

Table 6: Physical Light Emitting Diodes

| LED15 | LED14 | LED13 | LED12 | LED11 | LED10 | LED9 | LED8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| o_LED[15] | o_LED[14] | o_LED[13] | o_LED[12] | o_LED[11] | o_LED[10] | o_LED[9] | o_LED[8] |
| LED7 | LED6 | LED5 | LED4 | LED3 | LED2 | LED1 | LED0 |
| o_LED[7] | o_LED[6] | o_LED[5] | o_LED[4] | o_LED[3] | o_LED[2] | o_LED[1] | o_LED[0] |

# SECTION 4: TOP LEVEL IMPLEMENTATION

## 4.1: DESCRIPTION

The top level of the design is to implements the connection between the SOPC_CORE and the TSI. The SOPC_CORE contains the microprocessor and UART engine program. The program inside the microprocessor communicates with the SOPC_CORE through UART. The TSI contains references to the Artix 7 FPGA libraries and buffers all incoming input and output data from the hardware and software. I/O, including clock and reset, go through a buffer that is the TSI before heading out or into the SOPC_CORE. Input go through an IBUF or IBUFG and output go through an OBUF. Output signals are driven from the FPGA to external devices.

Figure 6: Top Level Block Diagram

## 4.2: BLOCK DIAGRAM

Figure 7: Top Level Detailed Diagram

## 4.3: DATA FLOW DESCRIPTION

The baudm is an input that determines the baud rate or transfer rate of data to and from the hardware. LEDs receive data to light up.

## 4.4: INPUT/OUTPUT

### 4.4.1: SIGNAL NAMES

Table 7: Top_Level Signal Names

| Signal | From | To |
| --- | --- | --- |
| SYS_CLK | 100MHz Crystal Oscillator | SOPC_CORE |
| SYS_RESET | BTNU | SOPC_CORE |
| i_SW | FPGA Switches | SOPC_CORE |
| i_RX | FPGA RX | SOPC_CORE |
| o_TX | SOPC_CORE | FPGA TX |
| [15:0] o_LED | SOPC_CORE | FPGA LEDs |

### 4.4.2: PIN ASSIGNMENTS

Table 8: Top_Level Pin Assignments

| Signal Name | Pin Location | Signal Name | Pin Location |
| --- | --- | --- | --- |
| SYS_CLK | E3 | o_LED[3] | N14 |
| SYS_RST | M18 | o_LED[4] | R18 |
| i_SW[1] | J15 | o_LED[5] | V17 |
| i_SW[2] | L16 | o_LED[6] | U17 |
| i_SW[3] | M13 | o_LED[7] | U16 |
| i_SW[4] | R15 | o_LED[8] | V16 |
| i_SW[5] | R17 | o_LED[9] | T15 |
| i_SW[6] | T18 | o_LED[10] | U14 |
| i_SW[7] | U18 | o_LED[11] | T16 |
| o_TX | D4 | o_LED[12] | V15 |
| i_RX | C4 | o_LED[13] | V14 |
| o_LED[0] | H17 | o_LED[14] | V12 |
| o_LED[1] | K15 | o_LED[15] | V11 |
| o_LED[2] | J13 |  |  |

### 4.4.3: ELECTRICAL CHARACTERISTICS

- Buttons

  - 3.3V as logical 1

  - 0V as logical 0

- Switches

  - 1.8V as logical 1

  - 0V as logical 0

## 4.5: CLOCKS

The Nexys 4 Artix-7 FPGA board has a single 100MHz crystal oscillator clock.

## 4.6: RESETS

Every register and flop within the SOPC utilize a single reset that is generated by a synchronized signal through the use of an asynchronous in synchronous out (AISO) hardware model. This is to prevent any possibility of metastability happening during reset due to unsynced signals.

## 4.7: SOFTWARE

### 4.7.1: DESCRIPTION

The assembly program that the microprocessor executes is programmed into the PROM. The program is an interface to allow the UART protocol to communicate with other devices outside of the SOPC. The program first does a memory test on bootup and is followed by a banner. After the banner a prompt is initiated to allow the user to input characters from the keyboard. Any key press will echo back what the user input. There are specific characters with added functions when received by the UART. The ‘@’ key displays the number of characters received since reset, the ‘*” key displays the author’s hometown and the ‘%’ key dumps the memory again. The memory takes in all received data and stores them inside the embedded RAM to be accessed and dumped. Only on hardware reset will cause the embedded RAM to be emptied.

### 4.7.2: SOFTWARE PLANNING DOCUMENT

Figure 8: Flowchart for Initialization and Main Loop

Pseudo Code for ISR:
Start @ Address 0500
read status flag
obtain TXRDY flag and check if high
if TXRDY flag is high
	call TXRDY function
obtain RXRDY flag and check if high
if RXRDY flag is high
	call RXRDY function

Pseudo Code for TXRDY Function:
if DISPLAY register is 8
	jump to MEM_TEST0 function
else if 9
	jump to MEM_TEST1 function
else if A
 	jump to MEM_TEST2 function
else if B
	jump to MEM_TEST3 function
else if C
	jump to MEM_TEST4 function
else if D
	jump to MEM_TEST5 function
else if DISPLAY register is 1
 	jump to BANNER_O function
else if DISPLAY register is 2
 	jump to PROMPT_O function
else if DISPLAY register is 3
 	jump to HOMETOWN_O function
else if DISPLAY register is 4
 	jump to CHAR_COUNT_O function
else if DISPLAY register is 5
 	jump to BACKSPACE_O function
else if DISPLAY register is 6
 	jump to CAR_RET_O function
else if DISPLAY register is 7
 	jump to MEM_DUMP function
fetch from memory pointed by stack pointer
output memory contents @location
increment stack pointer

Pseudo Code for RXRDY Function:
if STATUS is equal to (Status & 1C) 
	call ERROR function
check RX input
if RX is null
	return to ISR
if RX is the same as RX old
	jump to RXFB
if RX is hex 2A (asterisk)
	set DISPLAY to 3
 	set stack pointer to 9
else if RX is hex 40 (@)
 	convert character count binary to ascii and store into memory
	set DISPLAY to 4
 	set stack pointer to hex F0
else if RX is hex 7F (backspace)
	if character counter is 0
 		return to ISR
	set DISPLAY to 5
 	set stack pointer to hex 16
 	subtract from character counter
 	subtract from SRAM pointer
else if RX is hex D (carriage return)
	set DISPLAY to 6
 	set stack pointer to hex 14
else if RX is hex 25 (percent)
	set DISPLAY to 7
 	set stack pointer to hex 8000
else
	increment character counter
	output RX to SRAM
	increment SRAM pointer
	output RX to terminal
	set RX old to RX
	jump to CAR_RET_I
return to ISR

Pseudo Code for Memory Test:
initialize stack pointer
initialize DISPLAY register to 8
load temporary register with AAAA
output to memory address pointed by stack pointer
increment stack pointer
output content at location in memory
reset stack pointer to initialized value
repeat with 5555
initialize stack pointer
load address location into temporary register
output address to memory address pointed by stack pointer
increment stack pointer
output content at location in memory
reset stack pointer to 0
load DISPLAY register to 0
return to ISR

Pseudo Code for BANNER_O Function:
for stack pointer < 11
 	output from memory location pointed by stack pointer
set stack pointer to 0
store string characters of prompt onto stack starting at stack point 0
store string characters of hometown onto stack
store string characters of carriage return and line feed onto stack
store string characters of backspace onto stack
set DISPLAY reg to 0
return to ISR

Pseudo Code for PROMPT_O Function:
for stack pointer < A
 	output from memory location pointed by stack pointer
set stack pointer to 0
set DISPLAY reg to 0
return to ISR

Pseudo Code for HOMETOWN_O Function:
for stack pointer < 16
 	output from memory location pointed by stack pointer
set stack pointer to 0
set DISPLAY reg to 0
return to ISR

Pseudo Code for CHAR_COUNT_O Function:
for stack pointer < 57
 	output from memory location pointed by stack pointer
set stack pointer to 0
set DISPLAY reg to 6
return to ISR

Pseudo Code for BACKSPACE_O Function:
for stack pointer < 19
 	output from memory location pointed by stack pointer
set stack pointer to 0
set temporary register = RX
set DISPLAY reg to 0
return to ISR

Pseudo Code for CAR_RET_O Function:
for stack pointer < 16
 	output from memory location pointed by stack pointer
set stack pointer to 0
set DISPLAY reg to 2
return to ISR

Pseudo Code for DUMP_MEM_O Function:
for stack pointer < SRAM pointer value
 	input from SRAM memory
	output from memory location pointed by stack pointer 
set stack pointer to 0
set DISPLAY reg to 0
return to ISR

### 4.7.3: SOURCE CODE

See A.14: ASSEMBLY PROGRAM

# SECTION 5: EXTERNALLY ACQUIRED BLOCKS

## 5.1: TRAMELBLAZE

### 5.1.1: DESCRIPTION

The TramelBlaze is a 16-bit emulator of the 8-bit PicoBlaze in order to use the PicoBlaze ISA in newer versions of the Nexys FPGA boards. It provides all the functions of the original PicoBlaze.

### 5.1.2: FUNCTIONAL REQUIREMENTS

The TramelBlaze requires the creation of the 128x16 Stack RAM and 512x16 Scratch RAM and then the use of an assembler to convert assembly code into a .coe file for the 4096x16 Instruction ROM to tell what the rest of the hardware to do. There is also a 16x16 array register.

### 5.1.3: BLOCK DIAGRAM

Figure 9: TramelBlaze Top Level Block Diagram

### 5.1.4: INPUT/OUTPUT

Table 9: TramelBlaze Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| CLK | [15:0] OUT_PORT |
| RESET | [15:0] PORT_ID |
| [15:0] IN_PORT | READ_STROBE |
| INTERRUPT | WRITE_STROBE |
|  | INTERRUPT_ACK |

# SECTION 6: INTERNALLY DEVELOPED BLOCKS

## 6.1: TSI

### 6.1.1: DESCRIPTION

The technology specific instantiation (TSI) is an I/O PAD. The I/O PADs interface with the pins of the device. They use the library of the device and serve as an input and output buffer of the device to external devices.

### 6.1.2: BLOCK DIAGRAM

Figure 10: TSI Block Diagram

### 6.1.3: INPUT/OUTPUT

Table 10: TSI Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| SYS_CLK | o_CLK |
| SYS_RESET | o_RESET |
| i_RX | o_RX |
| [7:0] i_SW | [7:0] o_SW |
| i_TX | o_TX |
| [15:0] i_LED | [15:0] o_LED |

### 6.1.4: SOURCE CODE

See A.2: TECHNOLOGY SPECIFIC INSTATIATIONS

## 6.2: SOPC_CORE

### 6.2.1: DESCRIPTION

The SOPC_CORE exists to implement a processor that receives and transfers bits of data. The current design implements the full Universal Asynchronous Receiver Transmitter (UART) and works in unison with the TramelBlaze to receive and transmit data via terminal. The switches of the UART controls the baud rate at which data is being transferred and received and the parity control.

The SOPC_CORE is made up of the UART RX and TX Engine, TramelBlaze, Baud Decoder, Address Decoder, Asynchronous-In-Synchronous-Out Register, Positive Edge Detect, LEDs, SR flop and load register.

Figure 11: SOPC_CORE Block Design

SOPC_CORE

4

16

uart_rxd_out

baudm

bit8

leds

pen

ohel

clk100mhz

reset

uart_txd_in

rx_engine – This is the UART Receive Engine. It receives data bits input by the user. It generates interrupts to the microprocessor and the amount and speed of data bits it receives are changed based on the parity control and baud switches.

tx_engine – This is the UART Transmit Engine. It transmits data bits, generates interrupts to the microprocessor and the amount and speed of data bits transmitted are changed based on the parity control and baud switches.

TramelBlaze – A 16-bit microprocessor that is emulating an 8-bit microprocessor PicoBlaze. The microprocessor is externally developed by John Tramel to run and execute program-based assembly instructions from the 4K x 16 PROM.

baud_dec – This is the baud rate decoder. It has eleven types of baud rate to select from based on switch [7:4] of the FPGA starting from the slowest to fastest.

AISO_register – The Asynchronous-In-Synchronous-Out register allows an input signal to become synchronous across the entire hardware. This register is used to bring reset to a synchronous known state in order to prevent metastability.

posedge_detect – This allows an active signal to be caught for 1 clock cycle and no more than that.

LEDs – The LED register is assigned the output of the contents of specific register of the program depending on a specific write address.

SR_flop – The set and reset flip flop waits for the pulse signal from the posedge_detect to acknowledge the interrupt request and turn off the interrupt.

LD_reg – This register receives a load signal from a specific write address of the program from TramelBlaze and assigns the UARTCONFIG signal the state of the switches from the FPGA.

### 6.2.2: BLOCK DIAGRAM

Figure 12: SOPC_CORE Detailed Block Design

### 6.2.3: INPUT/OUTPUT

Table 11: SOPC_CORE Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk100mhz | uart_rxd_out |
| reset | [15:0] leds |
| bit8 |  |
| pen |  |
| ohel |  |
| [3:0] baudm |  |
| uart_txd_in |  |

### 6.2.4: VERIFICATION

See section 7.1: SOPC CORE VERIFICATION

### 6.2.5: SOURCE CODE

See A.3: SOPC CORE

## 6.3: RECEIVE ENGINE

### 6.3.1: DESCRIPTION

The purpose of this module is to provide half of the Universal Asynchronous Receiver Transmitter (UART). The focus will be the receive. The receive engine consists of two module components that make it up. It has the RX Control Unit and RX Datapath. The RX Control comprises the counter values to make sure the speed at which bits are received are the same as the baud rate. The RX Datapath uses the counter to make sure the RX data in are received at the right time and checks for when it is ready to receive the next set of data, parity error, framing error and overflow error.

Figure 13: RX_Engine Block

### 6.3.2: BLOCK DIAGRAM

Figure 14: Receive Engine Detailed Block Design

### 6.3.3: INPUT/OUTPUT

Table 12: RX Engine Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | rx |
| reset | rxrdy |
| baud_rate [19:0] | perr |
| read | ferr |
| bit8 | ovf |
| pen |  |
| ohel |  |

### 6.3.4: VERIFICATION

See section 7.2: RECEIVE ENGINE VERIFICATION

### 6.3.5: SOURCE CODE

See A.11: RECEIVE ENGINE

### 6.3.6: RECEIVE ENGINE CONTROL UNIT

#### 6.3.6.1: Definition

The receive engine control unit generates the control signals to the receive engine datapath. The control unit has a bit time counter that counts to the full or half bit time value of the baud rate according to which state it is in the Finite State Machine (FSM). In state 1 or the start state, the time is half because the state machine is checking to see if the start bit remains for half a bit time.

There is a bit counter, which counts the frame of the data selected by eight bit and parity enable bit.

The FSM consists of 3 states. For the first state, it is consistently checking for a transition from high to low of the RX before going to state 1 in order to find when to get the start bit. Start then remains high for half a bit time before moving to the last state. It remains in state 2 until done is set and starts over from state 0.

#### 6.3.6.2: Finite State Machine

Figure 15: RX_control FSM

#### 6.3.6.3: Input/Output

Table 13: RX_control Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | btu |
| reset | done |
| Rx | start |
| [19:0] baud |  |
| bit8 |  |
| pen |  |

#### 6.3.6.4: Verification

The module starts in state 0 before going into state 1 as soon as rx is low. Start and doit are set as soon as they reach state 1.

Figure 16: RX_control verification 1

It remains in state 1 until btu is high and rx remaining low.

Figure 17: RX_control verification 2

To go back to state 0, the signal done must be set. Doit remains high and start has already turned off at this point. As soon as done is set, the next state becomes 0 and the cycle is repeated.

Figure 18: RX_control verification 3

#### 6.3.6.5: Source Code

See A.12: RECEIVE ENGINE CONTROL UNIT

### 6.3.7: RECEIVE ENGINE DATAPATH

#### 6.3.7.1: Description

The receive engine datapath receives the frame of bits from the UART RX to the shift register within and continues until it receives the stop bit. The bits inside the shift register are then remapped to 10 bits. 8 bits are transferred as data output. The UART status flags are also set in this module.

The RXRDY flag waits for when done is set before going high.

The PERR, or parity-bit error, flag is set when the data frame is mismatched with the parity bit set by the parity control.

The FERR, or framing error, flag is set when the stop bit is mismatched with the settings of parity control.

The OVF, or overflow error, flag is set when more data is received when the frame is already finished.

Figure 19: RX_DATAPATH Block Diagram

#### 6.3.7.2: Block Diagram

Figure 20: RX_Datapath Detailed Block Design

#### 6.3.7.3: Input/Output

Table 14: RX_Datapath Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| Clk | [7:0] toTB |
| Reset | rxrdy |
| Btu | perr |
| Start | ferr |
| bit8 | ovf |
| Pen |  |
| Ohel |  |
| Done |  |
| Read |  |
| SDI |  |

#### 6.3.7.4: Verification

This verification confirms that RXRDY is set when done is high. It also shows that FERR will set because done and the stop bit are high as well.

Figure 21: RX_Datapath Verification 1

PERR is set when pen, done and the par OR par_even are all high.

Figure 22: RX_Datapath Verification 2

OVF is set when rxrdy and done are active at the same time.

Figure 23: RX_Datapath Verification 3

#### 6.3.7.5: Source Code

See A.13: RECEIVE ENGINE DATAPATH

#### 6.3.7.6: Receive Engine Shift Register

##### 6.3.7.6.1: Definition

The RX shift register takes in incoming data bits from the UART RX. When bit time up is set, the bits start shifting until start is set. The first bit in is the first out.

##### 6.3.7.6.2: Block Diagram

Figure 24: RX Shift Register Detailed Block Diagram

##### 6.3.7.6.3: Input/Output

Table 15: RX Shift Register Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | [9:0] shift_reg |
| rst |  |
| SH |  |
| SDI |  |

##### 6.3.7.6.4: Verification

The simulation shows that the bits have been shifting in from MSB to LSB whenever SH is high in the figure below.

Figure 25: RX Shift Register Verification

##### 6.3.7.6.5: Source Code

See A.13: RECEIVE ENGINE DATAPATH

#### 6.3.7.7: Receive Engine Remap

##### 6.3.7.7.1: Definition

The remap is a combinational block used to re-position the data from the RX shift register depending on the parity control bits. This is to ensure the data frame is can be used to detect framing, parity and overflow error.

##### 6.3.7.7.2: Block Diagram

Figure 26: Remap Register Detailed Block Diagram

##### 6.3.7.7.3: Input/Output

Table 16: Remap Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| bit8 | [9:0] remap |
| pen |  |
| [9:0] data |  |

##### 6.3.7.7.4: Verification

While bit8 and pen are inactive, the data from the shift register is re-position 2 bits to the right because there are only 8 bits. When bit8 is set, it is re-position 1 bit to the right because now there is 9 bits. With both bit8 and pen there is 10 bits so no re-position.

Figure 27: Remap Verification

##### 6.3.7.7.5: Source Code

See A.13: RECEIVE ENGINE DATAPATH

## 6.4: TRANSMIT ENGINE

### 6.4.1: DESCRIPTION

The purpose of this module is to provide half of the Universal Asynchronous Receiver Transmitter (UART). The focus will be the transmit. The transmit engine consists of a shift register that shifts out one bit of data at a time to be transmitted to a terminal. There will be a baud rate controller to adjust the speed in which the data is transmitted. The TX engine uses a bit time counter to count to the baud rate in order to change the transfer speed. The bit counter lets the engine know when it is done. When this happens RS flip flops output bit 0 and TXRDY is asserted to prepare for the next TX. The parity decoder gives the two most significant bits to the SR if the SR wants it. It depends on the input of eight, pen and ohel.

Figure 28: TX_ENGINE Block Diagram

### 6.4.2: BLOCK DIAGRAM

Figure 29: TX_ENGINE Detailed Block Diagram

### 6.4.3: INPUT/OUTPUT

Table 17: TX_ENGINE Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | tx |
| reset | txrdy |
| [19:0] baud_rate |  |
| [7:0] out_port |  |
| bit8 |  |
| pen |  |
| ohel |  |

### 6.4.4: VERIFICATION

See section 7.3: TRANSMIT ENGINE VERIFICATION

### 6.4.5: SOURCE CODE

See A.10: TRANSMIT ENGINE

### 6.4.6: SHIFT REGISTER

#### 6.4.6.1: Definition

The shift register serially shifts 11-bit encoded data depending on the parity-bit controls from the parity-bit encoder. After shifting data internally, the shift register transmits LSB to MSB at the frequency of the baud rate. Transmit bit remains high until the start bit is sent.

Figure 30: Shift Register Block Diagram

#### 6.4.6.2: Block Diagram

Figure 31: Shift Register Detailed Block Diagram

#### 6.4.6.3: Input/Output

Table 18: Shift Register Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | SDO |
| reset |  |
| LD |  |
| SH |  |
| SDI |  |
| pen |  |
| ohel |  |
| eight |  |
| [7:0] ldata |  |

#### 6.4.6.3: Verification

See section 7.3: TRANSMIT ENGINE VERIFICATION

#### 6.4.6.4: Source Code

See A.10: TRANSMIT ENGINE

## 6.5: EMBEDDED STATIC RAM

### 6.5.1: DESCRIPTION

The 32K x 16 Embedded Static RAM takes up half of the memory space of the microprocessor. The SRAM stores every received byte from the UART and is never cleared. Immediately after the program is ran, a memory test is done and the memory goes through all the addresses from 0x8000 to 0xFFFF and tests it with repeating A’s, then 5’s, then the address and the address values. After that, the program runs normally. To do a memory dump, the program waits for a hex 25 or the % character to dump the memory. The contents in memory continue to increase every time a new byte is received.

### 6.5.2: BLOCK DIAGRAM

Figure 32: SRAM Block Diagram

### 6.5.3: INPUT/OUTPUT

Table 19: SRAM Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clka | douta |
| wea |  |
| dina |  |
| addra |  |

### 6.5.4: VERIFICATION

A verification is done to see if the memory takes in values to check for bit errors in the memory. The memory before reset is in an unknown state is shown in the figure below.

Figure 33: SRAM Verification 1

Memory is then filled with zeros after reset is shown in the figure below.

Figure 34: SRAM Verification 2

In the figures below, the memory is being filled with repeating A’s first in Figure 35 and followed by repeating 5’s in Figure 36 to test if any bits are stuck.

| Figure 35: SRAM Verification 3 | Figure 36: SRAM Verification 4 |
| --- | --- |

Lastly, the memory locations in the SRAM are being filled with their corresponding address value in Figure 37 below. This shows that the memory test is a success and the SRAM works.

Figure 37: SRAM Verification 5

## 6.6: ASYNCHRONOUS-IN SYNCHRONOUS-OUT

### 6.6.1: DEFINITION

This module is used to bring the entire system into a known state at the same edge of the clock. This prevents unexpected results and metastability. This is done incase system reset de-asserts itself in the middle of the clock pulse so the synchronized reset signal will de-assert itself on the next active edge of the clock.

### 6.6.2: BLOCK DIAGRAM

Figure 38: AISO Block Detailed Diagram

### 6.6.3: INPUT/OUTPUT

Table 20: AISO Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | reset_sync |
| reset |  |

### 6.6.4: VERIFICATION

The figure below shows that when system reset is pressed, the synchronized reset will de-assert itself at the active edge of the clock.

Figure 39: AISO Verification

### 6.6.5: SOURCE CODE

See A.4: ASYNCHRONOUS-IN SYNCHRONOUS-OUT REGISTER

## 6.7: LOAD REGISTER

### 6.7.1: DEFINITION

This register takes in the out port signal from the TramelBlaze when the program is updating the switch configurations. It will only update output when that happens.

### 6.7.2: BLOCK DIAGRAM

Figure 40: Load Register Block Detailed Diagram

### 6.7.3: INPUT/OUTPUT

Table 21: Load Register Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | [7:0] UARTCONFIG |
| reset |  |
| Ld |  |
| [7:0] out_port |  |

### 6.7.4: VERIFICATION

The figure below shows that the output will only change when the load is asserted.

Figure 41: Load Register Verification

### 6.7.5: SOURCE CODE

See A.8: LOAD REGISTER

## 6.8: POSEDGE DETECT

### 6.8.1: DEFINITION

This module is used to capture a signal for one clock cycle. It is necessary in case a signal stays on too long and causes a specific hardware to do more than it is supposed to because of a long signal.

### 6.8.2: BLOCK DIAGRAM

Figure 42: Posedge Detect Detailed Block Diagram

### 6.8.3: INPUT/OUTPUT

Table 22: Posedge Detect Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | POS_detect |
| reset |  |
| INC_db_in |  |

### 6.8.4: VERIFICATION

The figure below shows that the PED captures a signal for one clock cycle even though signal remains on.

Figure 43: Posedge Detect Verification

### 6.8.5: SOURCE CODE

See A.7: POSITIVE-EDGE DETECT

## 6.9: ADDRESS DECODER

### 6.9.1: DEFINITION

This combination block decodes the port ID from the TramelBlaze. It uses a low active port ID [15] as an enable. When read or write strobe is high, the bit position is the current write or read output index. If port ID is hex 0, then output[0] is used.

### 6.9.2: BLOCK DIAGRAM

Figure 44: Address Decoder Block Diagram

### 6.9.3: INPUT/OUTPUT

Table 23: Address Decoder Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| [15:0] port_id | wr |
| writestrobe | rd |
| readstrobe |  |

### 6.9.4: VERIFICATION

When the port ID is hex 1 and write strobe is high, the write output reads hex 2. When the port ID is hex 2 and read strobe is high, the read output reads hex 4.

Figure 45: Address Decoder Verification

### 6.9.5: SOURCE CODE

See A.5: ADDRESS DECODER

## 6.10: BAUD DECODER

### 6.10.1: DEFINITION

This combination block determines the baud rate frequency for the UART protocol to allow communication with external devices. Frequencies must match in order to receive or transmit data bits correctly.

Figure 46: Baud Decoder Block Diagram

| baud_mode [7:4] | Bit Time | Baud Rate | Bit Time Count |
| --- | --- | --- | --- |
| 0000 | 3.3333 ms | 300 | 333,333 |
| 0001 | 833.33 us | 1200 | 83,333 |
| 0010 | 416.66 us | 2400 | 41,667 |
| 0011 | 208.33 us | 4800 | 20,833 |
| 0100 | 104.16 us | 9600 | 10,417 |
| 0101 | 52.083 us | 19200 | 5,208 |
| 0110 | 26.041 us | 38400 | 2,604 |
| 0111 | 17.361 us | 57600 | 1,736 |
| 1000 | 8.6806 us | 115200 | 868 |
| 1001 | 4.3403 us | 230400 | 434 |
| 1010 | 2.1701 us | 460800 | 217 |
| 1011 | 1.0851 us | 921600 | 109 |

### 6.10.2: INPUT/OUTPUT

Table 24: Baud Decoder Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| [4:0] baud_mode | [19:0] baud_rate |

### 6.10.3: VERIFICATION

The baud mode switches do match up with the bit time count value for the baud rates in the figure below.

Figure 47: Baud Decoder Verification

### 6.10.4: SOURCE CODE

See A.9: BAUD DECODER

## 6.9: SR FLOP

### 6.11.1: DEFINITION

The set reset flip-flop’s purpose is to send an interrupt request to the TramelBlaze when set is high. Set gets its signal from TXRDY and RXRDY. Reset is active when the TramelBlaze acknowledges the interrupt.

### 6.11.2: BLOCK DIAGRAM

Figure 48: SR Flop Block Diagram

### 6.11.3: INPUT/OUTPUT

Table 25: SR Flop Signal Names

| INPUTS | OUTPUTS |
| --- | --- |
| clk | intr |
| sys_rst |  |
| reset |  |
| set |  |

### 6.11.4: VERIFICATION

The figure below shows that interrupt only turns on when set turns high. It remains high until reset is asserted. It won’t turn on even if set is active while reset is high.

Figure 49: SR Flop Verification

### 6.11.5: SOURCE CODE

See A.6: SET RESET FLOP

# SECTION 7: CHIP VERIFICATION PLAN

## 7.1: SOPC CORE VERIFICATION

To prove that the SOPC_CORE works with the TramelBlaze, this simulation log file shows that the banner and prompt of the program is being received. It receives a data bits and transmits them. In this case CSULB 460 is the banner and thomas:~$ is the prompt.

Isim log file Isim P.20131013 (signature 0x7708f090) This is a Full version of Isim. Time resolution is 1 ps # onerror resume # wave add / # run 1000 ns Simulator is doing circuit initialization process. Finished circuit initialization process. Serial data transmitted=000000000000 data transmitted=00 Initial leds=0000000000000000 New Data Byte Incoming for i= 0 {EIGHT,PEN,OHEL}=000 Incoming bit = # run all PTR is 0000 leds=0000000000000001 1Serial data transmitted=011100000000 data transmitted=00 PTR is 0000 0Serial data transmitted=011100000000 data transmitted=00 PTR is 0001 0Serial data transmitted=011110000110 data transmitted=43 C PTR is 0002 0Serial data transmitted=011110100110 data transmitted=53 S PTR is 0003 00Serial data transmitted=011110101010 data transmitted=55 U PTR is 0004 0Serial data transmitted=011110011000 data transmitted=4c L PTR is 0005 0Serial data transmitted=011110000100 data transmitted=42 B PTR is 0006 0Serial data transmitted=011101000000 data transmitted=20 PTR is 0007 1Serial data transmitted=011110000110 data transmitted=43 C PTR is 0008 11Serial data transmitted=011110001010 data transmitted=45 E PTR is 0009 1Serial data transmitted=011110000110 data transmitted=43 C PTR is 000a 1Serial data transmitted=011110100110 data transmitted=53 S PTR is 000b 1Serial data transmitted=011101000000 data transmitted=20 PTR is 000c 1Serial data transmitted=011101101000 data transmitted=34 4 PTR is 000d 1Serial data transmitted=011101101100 data transmitted=36 6 1PTR is 000e 1Serial data transmitted=011101100000 data transmitted=30 0 PTR is 000f 1Serial data transmitted=011100011010 data transmitted=0d PTR is 0010 1Serial data transmitted=011100010100 data transmitted=0a 1PTR is 0000 1Serial data transmitted=011111101000 data transmitted=74 t PTR is 0001 1 New Data Byte Incoming for i= 1 {EIGHT,PEN,OHEL}=001 Incoming bit = Serial data transmitted=011111010000 data transmitted=68 h PTR is 0002 1Serial data transmitted=011111011110 data transmitted=6f o 0PTR is 0003 1Serial data transmitted=011111011010 data transmitted=6d m PTR is 0004 0Serial data transmitted=011111000010 data transmitted=61 a PTR is 0005 0Serial data transmitted=011111100110 data transmitted=73 s PTR is 0006 0Serial data transmitted=011101110100 data transmitted=3a : PTR is 0007 0Serial data transmitted=011111111100 data transmitted=7e ~ 0PTR is 0008 0Serial data transmitted=011101001000 data transmitted=24 $ PTR is 0009 1Serial data transmitted=011101000000 data transmitted=20 leds=0000000000011000 DATA RECEIVED=1000000101 11111111111111

## 7.2: RECEIVE ENGINE VERIFICATION

When parity is set to 8N1 and the value is 01010101, the receive bit reads in bit in this order: 10101010. The stop bit as the last high signal of rx where done is high. Btu marks the k/2 of the parity bits. Start happens as soon as the mark goes low for k/2.

Figure 50: RX_Engine Verification

## 7.3: TRANSMIT ENGINE VERIFICATION

When using and observing the test bench file for the TX engine. We can find that for every time there is an out_port data, the shift register will then take in the first two bits of 1 and 0 and then the outport and an MSB bit. The shift register is a 11-bit register while the out_port is an 8-bit register. When pen is enabled, that is when parity bits matter.

Because the TX only triggers every time a bit is shifted out of the shift register, this simulation log is used to compare the values it takes from the shift register. They happen to be the same.

ISim log file Running: C:\Users\vshoot\Documents\CECS\cecs460\Lab2\Lab2\tx_engine_tb_isim_beh.exe -intstyle ise -gui -tclbatch isim.cmd -wdb C:/Users/vshoot/Documents/CECS/cecs460/Lab2/Lab2/tx_engine_tb_isim_beh.wdb ISim P.20131013 (signature 0x7708f090) This is a Full version of ISim. Time resolution is 1 ps # onerror resume # wave add / # run 1000 ns Simulator is doing circuit initialization process. ----------------------------------------------------------------- CECS 460 Lab 2: Transmit Engine Test Bench ----------------------------------------------------------------- Finished circuit initialization process. {bit8, pen, ohel} = 000 --- Out_Port = 10101010 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11010101001 TX = # run all 10010101011 NP bit {bit8, pen, ohel} = 001 --- Out_Port = 10101011 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11010101101 TX = 10110101011 NP bit {bit8, pen, ohel} = 010 --- Out_Port = 10101101 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 10010110101 TX = 10101101001 EP bit = 0 {bit8, pen, ohel} = 011 --- Out_Port = 10110000 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11011000001 TX = 10000011011 OP bit = 0 {bit8, pen, ohel} = 100 --- Out_Port = 10110100 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11011010001 TX = 10001011011 NP bit {bit8, pen, ohel} = 101 --- Out_Port = 10111001 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11011100101 TX = 10100111011 NP bit {bit8, pen, ohel} = 110 --- Out_Port = 10111111 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11011111101 TX = 10111111011 EP bit = 1 {bit8, pen, ohel} = 111 --- Out_Port = 11000110 TRANSMIT (LSB to MSB) and SHIFT Comparison: Shift = 11100011001 TX = 10011000111 OP bit = 1 Stopped at time : 96912 ns : File "C:/Users/vshoot/Documents/CECS/cecs460/Lab2/Lab2/tx_engine_tb.v" Line 79

You can see here, as soon as loaddi is active on the active edge of the clock, the shift register takes in the first two bits of 1 and 0 and then the rest of the out_port plus another 1 bit.

Figure 51: Transmit Engine Verification

# SECTION 8: CHIP LEVEL TEST

In the terminal images below, the banner and the prompt are shown. The prompt repeats itself after character limits hits 40 and it returns automatically, any time the return key is pressed, and any time the key @ or * is pressed. When the @ key is pressed, the program will do a carriage return and linefeed before displaying the number of characters received since reset. Any time the * key is pressed, “LONG BEACH” will be displayed. Verification 2 just shows that the backspace key can be erased any characters entered and also modify the inputs.

Figure 52: Program Verification 1

The last line of the two figures show that the backspace is working properly.

Figure 53: Program Verification 2

Here, the memory is tested. First repeated A’s are displayed, followed by repeated 5’s to show no bits are stuck and then the SRAM addresses are displayed after that. After the memory test, the banner and prompt are displayed normally. Then after every received input, the data is stored into the SRAM. To dump the SRAM, the ‘%’ character is inputted.

Figure 54: Memory Test 1

Repeating 5’s are done here and has proven that no bits have gotten stuck, therefore the SRAM is working properly.

Figure 55: Memory Test 2

The figure below shows the last memory test. This dumps all the address locations in the SRAM.

Figure 56: Memory Test 3

A memory dump test with % key. The % key is stored into the SRAM and dumped as well. A carriage return and linefeed are also stored in memory and affects the prompt by overwriting it sometimes. However, if no carriage returns and linefeed are done, everything will display correctly.

Figure 57: Dump Memory Test

# APPENDIX:

This section includes all the Verilog code for every module and the assembly program code for the chip design, except for external sources.

## A.1: TOP LEVEL

`timescale 1ns / 1ps

//****************************************************************//

//  File name: top_level.v                                        //

//                                                                //

//  Created by       Thomas Nguyen on 5/01/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module top_level(SYS_CLK, SYS_RST, i_RX, i_SW, o_TX, o_LED);

input         SYS_CLK, SYS_RST, i_RX;

input   [7:0] i_SW;

output        o_TX;

output [15:0] o_LED;

wire          w_CLK, w_RST, w_TX, w_RX;

wire    [7:0] w_SW;

wire   [15:0] w_LED;

TSI       tsi(.SYS_CLK(SYS_CLK), .SYS_RST(SYS_RST), .i_RX(i_RX),

.i_SW(i_SW),       .o_TX(o_TX),       .o_LED(o_LED),

.i_TX(w_TX),       .i_LED(w_LED),     .o_CLK(w_CLK),

.o_RST(w_RST),     .o_RX(w_RX),       .o_SW(w_SW));

SOPC_CORE cor(.clk100mhz(w_CLK), .reset(w_RST), .baudm(w_SW[7:4]),

.bit8(w_SW[3]),    .pen(w_SW[2]), .ohel(w_SW[1]),

.uart_txd_in(w_RX),

.uart_rxd_out(w_TX), .leds(w_LED));

endmodule

## A.2: TECHNOLOGY SPECIFIC INSTATIATIONS

`timescale 1ns / 1ps

//****************************************************************//

//  File name: top_level.v                                        //

//                                                                //

//  Created by       Thomas Nguyen on 5/01/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module TSI(SYS_CLK, SYS_RST, i_RX, i_SW, i_TX, i_LED,

o_CLK, o_RST, o_RX, o_SW, o_TX, o_LED);

input          SYS_CLK, SYS_RST, i_RX;

input    [7:0] i_SW;

input          i_TX;

input   [15:0] i_LED;

output         o_CLK, o_RST, o_RX;

output   [7:0] o_SW;

output         o_TX;

output  [15:0] o_LED;

//Input Buffer

IBUFG #(.IOSTANDARD("DEFAULT"))

SYSTEM_CLK(.O(o_CLK), .I(SYS_CLK));

IBUF #(.IOSTANDARD("DEFAULT"))

SYSTEM_RESET(.O(o_RST), .I(SYS_RST));

IBUF #(.IOSTANDARD("DEFAULT"))

RX(.O(o_RX), .I(i_RX));

IBUF #(.IOSTANDARD("DEFAULT"))

SWITCHES[7:1](.O(o_SW[7:1]), .I(i_SW[7:1]));

//Output Buffer

OBUF #(.IOSTANDARD("DEFAULT"))

TX(.O(o_TX), .I(i_TX));

OBUF #(.IOSTANDARD("DEFAULT"))

LED[15:0](.O(o_LED), .I(i_LED));

endmodule

## A.3: SOPC CORE

`timescale 1ns / 1ps

//****************************************************************//

//  File name: SOPC_CORE.v                                        //

//                                                                //

//  Created by       Thomas Nguyen on 5/01/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module SOPC_CORE(clk100mhz, reset, baudm, bit8, pen, ohel,

uart_txd_in, uart_rxd_out, leds);

input         clk100mhz, reset;

//Baud rate mode

input   [3:0] baudm;

//Status Flags

input         bit8, pen, ohel;

//Receive signal

input         uart_txd_in;

//Transmit signal

output        uart_rxd_out;

//16 LEDs

output [15:0] leds;

reg    [15:0] leds;

//Tramelblaze

wire   [15:0] inport, outport, portid;

wire          rst_sync;

wire          ped_d;

wire    [7:0] rx_out;

wire    [7:0] UARTCONFIG;

wire   [15:0] write_d, read_d, SRAM_out;

wire   [19:0] br;

// -------------------- INSTANTIATE MODULES -------------------------------

AISO_register       AISO(.clk(clk100mhz), .reset(reset), .reset_sync(rst_sync));

LD_reg                LR(.clk(clk100mhz), .reset(rst_sync), .ld(write_d[6]),

.d(outport[15:0]), .q(UARTCONFIG));

baud_dec            BAUD(.baud_mode(UARTCONFIG[7:4]), .baud_rate(br));

tx_engine             TX(.clk(clk100mhz), .reset(rst_sync), .ld(write_d[0]),

.bit8(UARTCONFIG[3]), .pen(UARTCONFIG[2]),

.ohel(UARTCONFIG[1]),

.baud_rate(br), .out_port(outport[7:0]),

.tx(uart_rxd_out), .txrdy(txrdy));

rx_engine             RX(.clk(clk100mhz), .reset(rst_sync), .rx(uart_txd_in),

.bit8(UARTCONFIG[3]), .pen(UARTCONFIG[2]),

.ohel(UARTCONFIG[1]), .br(br),

.rxrdy(rxrdy), .perr(perr), .ferr(ferr), .ovf(ovf),

.rx_out(rx_out), .read(read_d[0]));

posedge_detect     PEDTX(.clk(clk100mhz), .reset(rst_sync), .INC_db_in(txrdy),

.POS_detect(ped_tx));

posedge_detect     PEDRX(.clk(clk100mhz), .reset(rst_sync), .INC_db_in(rxrdy),

.POS_detect(ped_rx));

SR_flop              SRF(.clk(clk100mhz),                   .rst_sync(rst_sync),

.reset(interruptack),              .set(ped_d),

.interrupt(interrupt));

tramelblaze_top      TBT(.CLK(clk100mhz),                   .RESET(rst_sync),

.IN_PORT(inport),                  .INTERRUPT(interrupt),

.OUT_PORT(outport),                .PORT_ID(portid),

.WRITE_STROBE(writestrobe),

.READ_STROBE(readstrobe),

.INTERRUPT_ACK(interruptack));

SRAM 	        SRAM(.clka(clk100mhz),

.wea(portid[15] && writestrobe),   .addra(portid[14:0]),

.dina(outport),                    .douta(SRAM_out));

//TXRDY OR RXRDY combo

assign ped_d = ((ped_tx | ped_rx) == 1'b1);

//mux for inport

assign inport = (read_d[5]) ? {8'b0, baudm, bit8, pen, ohel, 1'b0}:

(read_d[1]) ? {11'b0, ovf, ferr, perr, txrdy, rxrdy}:

(read_d[0]) ? {8'b0, rx_out}:

SRAM_out;

//Address decoder for write and read

addr_dec             DEC(.portid(portid), .ws(writestrobe),  .wr(write_d),

.rs(readstrobe), .rd(read_d));

always@(posedge clk100mhz)

if(rst_sync)

leds <= 16'b0;

else if(write_d[2])

leds <= outport;

else

leds <= leds;

endmodule

## A.4: ASYNCHRONOUS-IN SYNCHRONOUS-OUT REGISTER

`timescale 1ns / 1ps

//****************************************************************//

//  File name: AISO_register.v                                    //

//                                                                //

//  Created by       Thomas Nguyen on 9/13/18    .                //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

/* Description:

* Asynchronous in and synchronous out for reset.

*/

module AISO_register(clk, reset, reset_sync);

input      clk, reset;

output     reset_sync;

reg        q, d_reg;

always@(posedge clk or posedge reset)

if(reset) begin

d_reg <= 1'b0;

q <= 1'b0;

end else begin

d_reg <= 1'b1;

q <= d_reg;

end

assign reset_sync = ~q;

endmodule

## A.5: ADDRESS DECODER

`timescale 1ns / 1ps

//****************************************************************//

//  File name: addr_dec.v                                         //

//                                                                //

//  Created by       Thomas Nguyen on 4/08/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module addr_dec(portid, ws, rs, wr, rd);

input             ws, rs;

input      [15:0] portid;

output reg [15:0] wr, rd;

always@(*) begin

wr = 16'b0;

rd = 16'b0;

if(~portid[15]) begin

wr[portid[2:0]] = ws;

rd[portid[2:0]] = rs;

end

else begin

wr = wr;

rd = rd;

end

end

endmodule

## A.6: SET RESET FLOP

`timescale 1ns / 1ps

//****************************************************************//

//  File name: top_level.v                                        //

//                                                                //

//  Created by       Thomas Nguyen on 1/29/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module SR_flop(clk, rst_sync, reset, set, interrupt);

input      clk, rst_sync, reset, set;

output     interrupt;

reg        intr;

always@(posedge clk, posedge rst_sync)

if(rst_sync)

intr <= 1'b0;

else

case({reset, set})

{1'b0, 1'b1}: intr <= 1'b1;

{1'b1, 1'b0}: intr <= 1'b0;

default:      intr <= intr;

endcase

assign interrupt = reset ? 1'b0 : intr;

endmodule

## A.7: POSITIVE-EDGE DETECT

`timescale 1ns / 1ps

//****************************************************************//

//  File name: posedge_detect.v                                   //

//                                                                //

//  Created by       Thomas Nguyen on 09/20/2018.                 //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

//  Description:

//  A pulse maker that detects the first rising edge and generates

//  a one clock pulse

////////////////////////////////////////////////////////////////////

module posedge_detect(clk, reset, INC_db_in, POS_detect);

input      clk, reset, INC_db_in;

output     POS_detect;

reg        q, d;

always@(posedge clk or posedge reset)

if(reset) begin

d <= 1'b0;

q <= 1'b0;

end else begin

d <= INC_db_in;

q <= d;

end

assign POS_detect = ~q & d;

endmodule

## A.8: LOAD REGISTER

`timescale 1ns / 1ps

//****************************************************************//

//  File name: LD_reg.v                                           //

//                                                                //

//  Created by       Thomas Nguyen on 3/26/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module LD_reg(clk, reset, ld, d, q);

input            clk, reset, ld;

input      [7:0] d;

output reg [7:0] q;

//Sequential Block. If load is set, output gets input else stays the same

always@(posedge clk, posedge reset)

if(reset)    q <= 8'b0;

else if(ld)  q <= d;

else         q <= q;

endmodule

## A.9: BAUD DECODER

`timescale 1ns / 1ps

//****************************************************************//

//  File name: baud_dec.v                                         //

//                                                                //

//  Created by       Thomas Nguyen on 3/17/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module baud_dec(baud_mode, baud_rate);

input       [3:0] baud_mode;

output reg [19:0] baud_rate;

// ------------ Get Baud Rate --------- //

always@(*)

case(baud_mode)

// ------------------ Count No. ------ Rate

4'b0000 : baud_rate = 20'd333_333; //   300

4'b0001 : baud_rate = 20'd083_333; //  1200

4'b0010 : baud_rate = 20'd041_667; //  2400

4'b0011 : baud_rate = 20'd020_833; //  4800

4'b0100 : baud_rate = 20'd010_417; //  9600

4'b0101 : baud_rate = 20'd005_208; // 19200

4'b0110 : baud_rate = 20'd002_604; // 38400

4'b0111 : baud_rate = 20'd001_736; // 57600

4'b1000 : baud_rate = 20'd000_868; //115200

4'b1001 : baud_rate = 20'd000_434; //230400

4'b1010 : baud_rate = 20'd000_217; //460800

default : baud_rate = 20'd000_109; //921600

endcase

endmodule

## A.10: TRANSMIT ENGINE

`timescale 1ns / 1ps

//****************************************************************//

//  File name: tx_engine.v                                        //

//                                                                //

//  Created by       Thomas Nguyen on 2/18/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module tx_engine(  clk, reset, ld, bit8, pen, ohel, baud_rate,

out_port, tx, txrdy);

input              clk, reset, ld, bit8, pen, ohel;

input        [7:0] out_port;

input       [19:0] baud_rate;    //Baud count value

output             tx;           //Transmit

//flip flops

output reg         txrdy;        //Transmit Ready

reg                doit, loaddi;

//registers

reg          [7:0] ldata;        //Ldata

reg         [10:0] shift;        //Shift Register

//counters

reg          [3:0] bc, bc_i;     //Bit Count Value

reg         [19:0] btc, btc_i;   //Bit Time Count Value

wire               done, btu;

reg          [1:0] parity;       //2 MSB for Shift Register

////////////COUNTERS//////////////////

//bit counter

assign done = (bc_i == 4'b1011);

always@(*)

if(done)        bc = 4'b0;

else if({doit, btu} == 2'b11)

bc = bc_i + 4'b1;

else if({doit, btu} == 2'b10)

bc = bc_i;

else            bc = 4'b0;

always@(posedge clk, posedge reset)

if(reset)       bc_i <= 4'b0;

else            bc_i <= bc;

//bit time counter

assign btu = (btc_i == baud_rate);

always@(*)

if(btu)         btc = 20'b0;

else if({doit, btu} == 2'b10)

btc = btc_i + 20'b1;

else            btc = 20'b0;

always@(posedge clk, posedge reset)

if(reset)       btc_i <= 20'b0;

else            btc_i <= btc;

// ------------ FLIP FLOPS ------------ //

//loaddi flop

always@(posedge clk, posedge reset)

if(reset)       loaddi <= 1'b0;

else            loaddi <= ld;

//RS flop called DOIT

always@(posedge clk, posedge reset)

if(reset)       doit <= 1'b0;

else if(done)   doit <= 1'b0;

else if(loaddi) doit <= 1'b1;

else            doit <= doit;

//TX bit

assign tx = shift[0];

//TXRDY RS flop (LOW ACTIVE)

always@(posedge clk, posedge reset)

if(reset)       txrdy <= 1'b1;

else if(ld)     txrdy <= 1'b0;

else if(done)   txrdy <= 1'b1;

else            txrdy <= txrdy;

// ------------ REGISTERS ------------ //

//ldata register

always@(posedge clk, posedge reset)

if(reset)       ldata <= 8'b0;

else if(ld)     ldata <= out_port;

else            ldata <= ldata;

//shift register

always@(posedge clk, posedge reset)

if(reset)       shift <= 11'h7ff;

else if(loaddi) shift <= {parity, ldata[6:0], 1'b0, 1'b1};

else if(btu)    shift <= {1'b1, shift[10:1]};

else            shift <= shift;

//2-bit data for parity

always@(*)

case({bit8, pen, ohel})

3'h2   : parity = {   1'b1,   (^ldata[6:0])};

3'h3   : parity = {   1'b1,  ~(^ldata[6:0])};

3'h4   : parity = {   1'b1,        ldata[7]};

3'h5   : parity = {   1'b1,        ldata[7]};

3'h6   : parity = { (^ldata),      ldata[7]};

3'h7   : parity = {~(^ldata),      ldata[7]};

default: parity =     2'b11;

endcase

endmodule

## A.11: RECEIVE ENGINE

`timescale 1ns / 1ps

//****************************************************************//

//  File name: rx_engine.v                                        //

//                                                                //

//  Created by       Thomas Nguyen on 3/26/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module rx_engine(clk, reset, rx, bit8, pen, ohel, br, rxrdy, perr,

ferr, ovf, rx_out, read);

input         clk, reset;

input         rx, bit8, pen, ohel, read;

input  [19:0] br;

output        rxrdy, perr, ferr, ovf;

output  [7:0] rx_out;

wire          btu, done, start;

rx_control           RXC(.clk(clk), .reset(reset), .rx(rx),

.baud(br), .start(start),

.bit8(bit8), .pen(pen), .done(done), .btu(btu));

rx_datapath          RXD(.clk(clk), .reset(reset), .btu(btu),

.start(start), .bit8(bit8), .pen(pen),

.ohel(ohel), .done(done), .SDI(rx),

.rxrdy(rxrdy), .perr(perr), .ferr(ferr),

.ovf(ovf), .toTB(rx_out), .read(read));

endmodule

## A.12: RECEIVE ENGINE CONTROL UNIT

`timescale 1ns / 1ps

//****************************************************************//

//  File name: rx_control.v                                       //

//                                                                //

//  Created by       Thomas Nguyen on 3/18/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module rx_control(clk, reset, rx, baud, start, bit8, pen, done, btu);

input         clk, reset;

input         rx;

input         bit8, pen;

input  [19:0] baud;

output        btu;

output        done;

//Present Output

output        start;

reg           start;

reg           doit;

reg     [1:0] state;

//Next Outputs

reg           nstart, ndoit;

reg     [1:0] nstate;

//Counters

reg     [3:0] bc, bc_i;     //Bit Count Value

reg    [19:0] btc, btc_i;   //Bit Time Count Value

reg     [3:0] num;

wire   [19:0] start_mux;

////////////COUNTERS//////////////////

//Bit Time Counter

assign btu = (btc_i == start_mux);

always@(*)

if(btu)         btc = 20'b0;

else if({doit, btu} == 2'b10)

btc = btc_i + 20'b1;

else            btc = 20'b0;

always@(posedge clk, posedge reset)

if(reset)       btc_i <= 20'b0;

else            btc_i <= btc;

//Bit Counter

assign done = (bc_i == num);

always@(*)

if(done)        bc = 4'b0;

else if({doit, btu} == 2'b11)

bc = bc_i + 4'b1;

else if({doit, btu} == 2'b10)

bc = bc_i;

else            bc = 4'b0;

always@(posedge clk, posedge reset)

if(reset)       bc_i <= 4'b0;

else            bc_i <= bc;

//Combination logic for Bit Counter to increment to

always@(*)

case({bit8, pen})

2'b00  : num = 4'h9;

2'b11  : num = 4'hB;

default: num = 4'hA;

endcase

//Start Select Mux for K and K/2

assign start_mux = (start) ? (baud >> 1) : baud;

//FINITE STATE MACHINE (FSM)

//Modified Moore to ensure that the start bit remains

//active for half a bit time.

always@(posedge clk, posedge reset)

if(reset) {state, start, doit} <= 4'b00_0_0;

else      {state, start, doit} <= {nstate, nstart, ndoit};

always@(*)

case(state)

2'b00  : {nstate, nstart, ndoit} = (rx)   ? 4'b00_0_0 :

4'b01_1_1 ;

2'b01  : {nstate, nstart, ndoit} = (rx)   ? 4'b00_0_0 :

(!btu) ? 4'b01_1_1 :

4'b10_0_1 ;

2'b10  : {nstate, nstart, ndoit} = (done) ? 4'b00_0_0 :

4'b10_0_1 ;

default: {nstate, nstart, ndoit} =          4'b00_0_0 ;

endcase

endmodule

## A.13: RECEIVE ENGINE DATAPATH

`timescale 1ns / 1ps

//****************************************************************//

//  File name: rx_datapath.v                                      //

//                                                                //

//  Created by       Thomas Nguyen on 3/18/19.                    //

//  Copyright c 2019 Thomas Nguyen. All rights reserved.          //

//                                                                //

//                                                                //

//  In submitting this file for class work at CSULB               //

//  I am confirming that this is my work and the work             //

//  of no one else. In submitting this code I acknowledge that    //

//  plagiarism in student project work is subject to dismissal.   //

//  from the class                                                //

//****************************************************************//

module rx_datapath(clk, reset, btu, start, bit8, pen, ohel, done,

SDI, rxrdy, perr, ferr, ovf, read, toTB);

input            clk, reset;

input            btu, start, bit8, pen, ohel, done, read;

//Serial Data Input

input            SDI;

//Output of remap to tramel blaze

output     [7:0] toTB;

//Status for recieve, parity error, framing error and overflow

output reg       rxrdy, perr, ferr, ovf;

reg        [9:0] shift_reg, remap;

//Mux Signals that are used to create load signals to Status Regs

wire             par, par_even, gen, SH;

reg              stop;

/////FOR SHIFT REGISTER//////

//Shift signal

assign SH = btu & ~start;

always@(posedge clk, posedge reset)

if(reset)   shift_reg <= 10'b0;

else if(SH) shift_reg <= {SDI, shift_reg[9:1]};

else        shift_reg <= shift_reg;

/////FOR REMAP COMBO////////

assign toTB = (bit8) ? remap[7:0] : {1'b0, remap[6:0]};

always@(*)

case({bit8, pen})

2'b00  : remap = {2'b0, shift_reg[9:2]};

2'b01  : remap = {1'b0, shift_reg[9:1]};

2'b10  : remap = {1'b0, shift_reg[9:1]};

default: remap = shift_reg;

endcase

//Parity Gen Select

assign gen = (bit8) ? remap[7] : 1'b0;

//Even Parity Mux

assign par_even = (~ohel) ?  (remap[6:0] ^ gen) :

~(remap[6:0] ^ gen) ;

//Parity Bit Select

assign par = (bit8) ? remap[8] : remap[7];

//Stop Bit Select

always@(*)

case({bit8, pen})

2'b00  : stop = ~remap[7];

2'b11  : stop = ~remap[9];

default: stop = ~remap[8];

endcase

/////STATUS REGISTERS///////

//RXRDY REG

always@(posedge clk, posedge reset)

if(reset)             rxrdy <= 1'b0;

else if(read)         rxrdy <= 1'b0;

else if(done)         rxrdy <= 1'b1;

else                  rxrdy <= rxrdy;

//PERR REG

always@(posedge clk, posedge reset)

if(reset)             perr <= 1'b0;

else if(read)         perr <= 1'b0;

else if(done & (par ^ par_even) & pen)

perr <= 1'b1;

else                  perr <= perr;

//FERRR REG

always@(posedge clk, posedge reset)

if(reset)             ferr <= 1'b0;

else if(read)         ferr <= 1'b0;

else if(done & stop)  ferr <= 1'b1;

else                  ferr <= ferr;

//OVF REG

always@(posedge clk, posedge reset)

if(reset)             ovf <= 1'b0;

else if(read)         ovf <= 1'b0;

else if(done & rxrdy)

ovf <= 1'b1;

else                  ovf <= ovf;

endmodule

## A.14: ASSEMBLY PROGRAM

; Thomas Nguyen

; 016238935

; CECS 460

; SOPC TSI External Memory w/ UART

;

; Assembly code for UART communicating with an

; external memory outside of the MCU. The memory

; will be written to on every receive from UART.

; It can be accessed and get a memory dump by the use

; of the "%" character.

; ================================================

; Declare EQU constants

; ================================================

ZEROS  	    EQU     0000

ONE         EQU     0001

; ================================================

; Register EQU

; ================================================

SCRATCH     EQU     R0 ; Scratch RAM

SP          EQU     R1 ; Stack Pointer

SRAM        EQU     R2 ; SRAM pointer

TEMP        EQU     R3 ; Temp reg that takes Scratch mem

LEDS        EQU     R4 ;

STATUS      EQU     R5 ; Status register

COUNT       EQU     R6 ; Char Counter

DISPLAY     EQU     RA ; Keeps track of what is being displayed

RX          EQU     RB ; Received data bits

CONFIG      EQU     RC ; The config switch register

S_INDEX     EQU     RD ; Keep track of inputted strings

DELAY       EQU     RE ; Delays leds

DELAY2      EQU     RF ;

;Temp Reg for Bin to ASCII

RE          EQU     R7 ; Temp Reg for current count

RD          EQU     R8 ; Temp Reg for max count

RB          EQU     R9 ; Temp Reg for Adding

; ================================================

; Initialization

; ================================================

START       INPUT   CONFIG,     0005

OUTPUT  CONFIG,     0006

LOAD    COUNT,      ZEROS

LOAD    SP,         ZEROS

LOAD    CONFIG,     ZEROS

LOAD    LEDS,       ONE

LOAD    TEMP,       ZEROS

LOAD    STATUS,     ZEROS

LOAD    DISPLAY,    0008

LOAD    S_INDEX,    ZEROS

LOAD    DELAY,      ZEROS

LOAD    DELAY2,     ZEROS

LOAD    SRAM,       8000

; ================================================

; Initialize Scratch Memory with ASCII Hex values of characters from message

; ================================================

LOAD    SCRATCH,    0043    ; C

CALL    STORER

LOAD    SCRATCH,    0053    ; S

CALL    STORER

LOAD    SCRATCH,    0055    ; U

CALL    STORER

LOAD    SCRATCH,    004C    ; L

CALL    STORER

LOAD    SCRATCH,    0042    ; B

CALL    STORER

LOAD    SCRATCH,    0020    ; Space

CALL    STORER

LOAD    SCRATCH,    0043    ; C

CALL    STORER

LOAD    SCRATCH,    0045    ; E

CALL    STORER

LOAD    SCRATCH,    0043    ; C

CALL    STORER

LOAD    SCRATCH,    0053    ; S

CALL    STORER

LOAD    SCRATCH,    0020    ; Space

CALL    STORER

LOAD    SCRATCH,    0034    ; 4

CALL    STORER

LOAD    SCRATCH,    0036    ; 6

CALL    STORER

LOAD    SCRATCH,    0030    ; 0

CALL    STORER

LOAD    SCRATCH,    000D    ; <CR>

CALL    STORER

LOAD    SCRATCH,    000A    ; <LF>

CALL    STORER              ; Stored @ 0x0F, SP @ 0x10

LOAD    SP,         00F0    ;

LOAD    SCRATCH,    000D

CALL    STORER

LOAD    SCRATCH,    000A    ; <LF>

CALL    STORER

LOAD    SP,         8000   ; Set to Addr 8000 to prepare for mem_dump

ENINT

; ================================================

; MAIN LOOP for LEDs

; ================================================

MAIN        OUTPUT  LEDS,       0002

COMP    LEDS,       9000

JUMPNZ  LEDDELAY

INPUT   CONFIG,     0005

OUTPUT  CONFIG,     0006

LOAD    LEDS,       ONE

JUMP    MAIN

LEDDELAY    RL      LEDS

LOAD    DELAY,      FFFF

LOAD    DELAY2,     0004

COUNT_DWN   SUB     DELAY,      ONE

COMP,   DELAY,      ZEROS

JUMPNZ  COUNT_DWN

LOAD    DELAY,      FFFF

SUB     DELAY2,     ONE

COMP    DELAY2,     ZEROS

JUMPNZ  COUNT_DWN

JUMP    MAIN

; ================================================

; Bin to ASCII

; ================================================

ADDRESS 0200

BIN2ASCII   ADD     COUNT,      S_INDEX

LOAD    S_INDEX,    ZEROS

LOAD    RE,         COUNT

LOAD    RD,         2710    ; 10,000

CALL    FINDIT

ADD     RB,         0030

STORE   RB,         00F2

LOAD    RD,         03E8    ;  1,000

CALL    FINDIT

ADD     RB,         0030

STORE   RB,         00F3

LOAD    RD,         0064    ;    100

CALL    FINDIT

ADD     RB,         0030

STORE   RB,         00F4

LOAD    RD,         000A    ;     10

CALL    FINDIT

ADD     RB,         0030

STORE   RB,         00F5

LOAD    RD,         ONE

CALL    FINDIT

ADD     RB,         0030    ;      1

STORE   RB,         00F6

RETURN                      ; return to ISR

; ================================================

; Subroutines for Bin to ASCII

; ================================================

ADDRESS 0300

FINDIT      LOAD    RB,         ZEROS

FNDLOOP     SUB     RE,         RD      ; RE <- RE - RD

JUMPC   RESTORE             ; if carry jump

ADD     RB,         ONE

JUMP    FNDLOOP

RESTORE     ADD     RE,         RD      ; RE <- RE + RD

RETURN                      ; return to procedure

; ================================================

; Store to scratch ram and increment pointer

; ================================================

STORER      STORE   SCRATCH,    (SP)

ADD     SP,         ONE

RETURN

; ================================================

; Prompt (tomas:~$)

; ================================================

ADDRESS 0400

PROMPT      LOAD    SCRATCH,    0074    ; t

CALL    STORER              ; Stored @ 0x00

LOAD    SCRATCH,    0068    ; h

CALL    STORER

LOAD    SCRATCH,    006F    ; o

CALL    STORER

LOAD    SCRATCH,    006D    ; m

CALL    STORER

LOAD    SCRATCH,    0061    ; a

CALL    STORER

LOAD    SCRATCH,    0073    ; s

CALL    STORER

LOAD    SCRATCH,    003A    ; :

CALL    STORER

LOAD    SCRATCH,    007E    ; ~

CALL    STORER

LOAD    SCRATCH,    0024    ; $

CALL    STORER

LOAD    SCRATCH,    0020    ; Space

CALL    STORER              ; Stored @ 0x09

RETURN

; ================================================

; Home Town and Backspace (LONG BEACH<CR><LF><BS>)

; ================================================

HOMETOWN    LOAD    SCRATCH,    004C    ; L

CALL    STORER              ; Stored @ 0x0A

LOAD    SCRATCH,    004F    ; O

CALL    STORER

LOAD    SCRATCH,    004E    ; N

CALL    STORER

LOAD    SCRATCH,    0047    ; G

CALL    STORER

LOAD    SCRATCH,    0020    ; Space

CALL    STORER

LOAD    SCRATCH,    0042    ; B

CALL    STORER

LOAD    SCRATCH,    0045    ; E

CALL    STORER

LOAD    SCRATCH,    0041    ; A

CALL    STORER

LOAD    SCRATCH,    0043    ; C

CALL    STORER

LOAD    SCRATCH,    0048    ; H

CALL    STORER

LOAD    SCRATCH,    000D    ; <CR>

CALL    STORER              ; Stored @ 0x14

LOAD    SCRATCH,    000A    ; <LF>

CALL    STORER              ; Stored @ 0x15

LOAD    SCRATCH,    0008    ; Backspace

CALL    STORER              ; Stored @ 0x16

LOAD    SCRATCH,    0020    ; Space

CALL    STORER              ; 17

LOAD    SCRATCH,    0008    ; Backspace

CALL    STORER              ; Stored @ 0x18

LOAD    SP,         0D00

LOAD    SCRATCH,    ZEROS

CALL    STORER

LOAD    SP,         ZEROS   ; Set to Addr 0

RETURN

; ================================================

; Interrupt Service Routine

; ================================================

ADDRESS 0500

ISR         INPUT   STATUS,     ONE     ; read in status flag

AND     STATUS,     0002    ; obtain txrdy

COMP    STATUS,     0002    ; check if tx is high

CALLZ   TXRDY

INPUT   STATUS,     ONE

ISR2        COMP    DISPLAY     ZEROS

CALLZ   RXRDY

RETEN

; ================================================

; TX ready

; ================================================

TXRDY       COMP    Display,    0008    ;MemTest Start

JUMPZ   MEM_TEST0

COMP    Display,    0009

JUMPZ   MEM_TEST1

COMP    Display,    000A

JUMPZ   MEM_TEST2

COMP    Display,    000B

JUMPZ   MEM_TEST3

COMP    Display,    000C

JUMPZ   MEM_TEST4

COMP    Display,    000D    ;MemTest End

JUMPZ   MEM_TEST5

COMP    DISPLAY,    ONE

JUMPNZ  PROMPT_O

; ================================================

; Displays Banner

; ================================================

BANNER_O    COMP    SP,         0011

JUMPNZ  TX_OUT

LOAD    SP,         ZEROS

CALL    PROMPT

CALL    HOMETOWN

LOAD    DISPLAY,    0002

; ================================================

; Displays Prompt

; ================================================

PROMPT_O    COMP    DISPLAY,    0002

JUMPNZ  HOMETOWN_O

COMP    SP,         000A

JUMPNZ  TX_OUT

LOAD    TEMP,       RX

LOAD    DISPLAY,    ZEROS    ; finish with prompt message

; ================================================

; Displays Home town

; ================================================

HOMETOWN_O  COMP    DISPLAY,    0003

JUMPNZ  CHAR_CNT_O

COMP    SP,         0016

JUMPNZ  TX_OUT

LOAD    SP,         ZEROS

LOAD    DISPLAY,    0002

LOAD    TEMP,       ZEROS

OUTPUT  TEMP,       ZEROS

; ================================================

; Displays # of received characters

; ================================================

CHAR_CNT_O  COMP    DISPLAY,    0004

JUMPNZ  DUMP_MEM_O

COMP    SP,         00F7

JUMPNZ  TX_OUT

LOAD    SP,         0014

LOAD    DISPLAY,    0006

; ================================================

; Dumps contents of SRAM

; ================================================

DUMP_MEM_O  COMP    DISPLAY,    0007

JUMPNZ  BACKSPACE_O

INPUT   TEMP,       (SP)

OUTPUT  TEMP,       ZEROS

ADD     SP,         ONE

COMP    SP,         SRAM

JUMPC   TX_OUT

; ================================================

; Displays Backspace

; ================================================

BACKSPACE_O COMP    DISPLAY,    0005

JUMPNZ  CAR_RET_O

COMP    SP,         0019

JUMPNZ  TX_OUT

LOAD    SP,         ZEROS

LOAD    TEMP,       RX

LOAD    DISPLAY,    ZEROS

; ================================================

; Displays Carriage Return

; ================================================

CAR_RET_O   COMP    DISPLAY,    0006

RETURNNZ

COMP    SP,         0016

JUMPNZ  TX_OUT

LOAD    SP,         ZEROS

LOAD    DISPLAY,    0002

LOAD    TEMP,       ZEROS

OUTPUT  TEMP,       ZEROS

JUMP    EXIT_TX

; ================================================

; Transmits Characters through UART

; ================================================

TX_OUT      FETCH   TEMP,       (SP)    ; fetch from memory pointed by sp

OUTPUT  TEMP,       ZEROS   ; output memory content @loc

ADD     SP,         ONE     ; Increment through SP

EXIT_TX     RETURN

; ================================================

; Memory Testing; Display repeating A's, 5's & Address

; ================================================

MEM_TEST0   LOAD    SCRATCH,    AAAA

OUTPUT  SCRATCH,    (SP)

ADD     SP,         ONE

COMP    SP,         FFFF

JUMPNZ  MEM_TEST0

LOAD    SCRATCH,    ZEROS

OUTPUT  SCRATCH,    ZEROS

LOAD    SP,         8000

LOAD    DISPLAY,    0009

MEM_TEST1   INPUT   SCRATCH,    (SP)    ;Display A's

OUTPUT  SCRATCH,    ZEROS

ADD     SP,         ONE

COMP    SP,         FFFF

RETURNC

LOAD    SP,         8000

MEM_TEST2   LOAD    SCRATCH,    5555

OUTPUT  SCRATCH,    (SP)

ADD     SP,         ONE

COMP    SP,         FFFF

JUMPNZ  MEM_TEST2

LOAD    SP,         8000

LOAD    DISPLAY,    000B

MEM_TEST3   INPUT   SCRATCH,    (SP)    ;Display 5's

OUTPUT  SCRATCH,    ZEROS

ADD     SP,         ONE

COMP    SP,         FFFF

RETURNC

LOAD    SP,         8000

LOAD    SCRATCH,    ZEROS

LOAD    DISPLAY,    000C

MEM_TEST4   OUTPUT  SCRATCH,    (SP)    ;Load Address

ADD     SCRATCH,    ONE

ADD     SP,         ONE

COMP    SP,         FFFF

JUMPNZ  MEM_TEST4

LOAD    SP,         8000

OUTPUT  SCRATCH,    ZEROS

LOAD    DISPLAY,    000D

MEM_TEST5   INPUT   SCRATCH,    (SP)    ;Display Address

OUTPUT  SCRATCH,    ZEROS

ADD     SP,         ONE

COMP    SP,         FFFF

RETURNC

LOAD    SCRATCH,    ZEROS

LOAD    SP,         00F0

FETCH   TEMP,       (SP)

OUTPUT  TEMP,       ZEROS

ADD     SP,         ONE

FETCH   TEMP,       (SP)

OUTPUT  TEMP,       ZEROS

LOAD    TEMP,       ZEROS

LOAD    SP,         ZEROS

LOAD    DISPLAY,    ONE

RETURN

; ================================================

; RX ready

; ================================================

RXRDY       AND     STATUS,     001C

COMP    STATUS,     0000

CALLNZ  ERRORF

INPUT   RX,         ZEROS

COMP    RX,         ZEROS

JUMPZ   EXIT_RX

COMP    RX,         TEMP

JUMPZ   RXFB

COMP    RX,         002A    ; *

JUMPZ   HOMETOWN_I

COMP    RX,         0040    ; @

JUMPZ   CHAR_CNT_I

COMP    RX,         007F    ; Backspace

JUMPZ   BACKSPACE_I

COMP    RX,         000D    ; CR

JUMPZ   CAR_RET_I

COMP    RX,         0025    ; %

JUMPZ   DUMP_MEM

ADD     S_INDEX,    ONE

OUTPUT  RX,         ZEROS

OUTPUT  RX,         (SRAM)  ; Received Byte to SRAM

ADD     SRAM,       ONE

LOAD    TEMP,       RX

COMP    S_INDEX,    0028

JUMPZ   CAR_RET_I

EXIT_RX     RETEN

RXFB        LOAD    SP,         0D00

FETCH   RX,         (SP)

LOAD    TEMP,       RX

RETEN

; ================================================

; PERR, FERR, or OVF flags set

; ================================================

ERRORF      OUTPUT  STATUS,     0002    ; Output Error

RETURN

; ================================================

; Receive Home town input

; ================================================

HOMETOWN_I  LOAD    DISPLAY,    0003

LOAD    SP,         0009

JUMP    RESETSTRING

; ================================================

; Receive character count input

; ================================================

CHAR_CNT_I  CALL    BIN2ASCII

LOAD    DISPLAY     0004

LOAD    SP,         00F0

JUMP    RESETSTRING

; ================================================

; Receive Backspace

; ================================================

BACKSPACE_I COMP    S_INDEX,    ZEROS

JUMPZ   RXFB

LOAD    DISPLAY,    0005

LOAD    SP,         0016

SUB     S_INDEX,    ONE

SUB     SRAM,       ONE

LOAD    TEMP,       ZEROS

OUTPUT  TEMP,       ZEROS

RETEN

; ================================================

; Receive Carriage Return

; ================================================

CAR_RET_I   LOAD    DISPLAY,    0006

LOAD    SP,         0014

JUMP    RESETSTRING

; ================================================

; Start Dumping Memory

; ================================================

DUMP_MEM    LOAD    DISPLAY,    0007

LOAD    SP,         8000

JUMP    RESETSTRING

; ================================================

; Reset string index and output nothing

; ================================================

RESETSTRING ADD     COUNT,      S_INDEX

OUTPUT  RX,         (SRAM)  ; Received Byte to SRAM

ADD     SRAM,       ONE

LOAD    TEMP,       ZEROS

OUTPUT  TEMP,       ZEROS

LOAD    S_INDEX,    ZEROS

RETEN

; ================================================

; ISR vectored through 0FFE

; ================================================

ADDRESS 0FFE

ENDIT       JUMP    ISR

END

INTENTIONALLY BLANK